Nx586â Processor Recognition
Application Note

PRELIMINARY INFORMATION

, Incorporated.

1623 Buckeye Drive

Milpitas, CA 95035

ORDER # 754006-02

Copyright © 1994, 1995 by NexGen, Inc.

The goal of this databook is to enable our customers to make informed purchase decisions and to design systems around our described products.  Every effort is made to provide accurate information in support of these goals.  However, representations made by this databook are not intended to describe the internal logic and physical design.  Wherever product internals are discussed, the information should be construed as conceptual in nature.  No presumptions should be made about the internal design based on this document.  Information about the internal design of NexGen products is provided via nondisclosure agreement ("NDA") on a need to know basis.

The material in this document is for information only and is subject to change without notice.  NexGen reserves the right to make changes in the product specification and design without reservation and without notice to its users.  THIS DOCUMENT DOES NOT CONSTITUTE A WARRANTY OF ANY KIND WITH RESPECT TO THE NEXGEN INC. PRODUCTS, AND NEXGEN INC. SHALL NOT BE LIABLE FOR ANY ERRORS THAT APPEAR IN THIS DOCUMENT.

All purchases of NexGen products shall be subject to NexGen's standard terms and conditions of sale.  THE WARRANTIES AND REMEDIES EXPRESSLY SET FORTH IN SUCH TERMS AND CONDITIONS SHALL BE THE SOLE WARRANTIES AND THE BUYER'S SOLE AND EXCLUSIVE REMEDIES, AND NEXGEN INC. SPECIFICALLY DISCLAIMS ANY AND ALL OTHER WARRANTIES, WHETHER EXPRESS, IMPLIED OR STATUTORY, INCLUDING THE IMPLIED WARRANTIES OF FITNESS FOR A PARTICULAR PURPOSE, AGAINST INFRINGEMENT AND OF MERCHANTABILITY.  No person is authorized to make any other warranty or representation concerning the performance of the NexGen products.  In particular, NexGen's products are not specifically designed, manufactured or intended for sale as components for the planning, design, construction, maintenance, operation or use of any nuclear facility or other ultra-hazardous activity, and neither NexGen nor its suppliers shall have any liability with respect to such use

Trademark Acknowledgments

Nx586 is a registered trademark of NexGen, Inc..  NexGen, Nx686, RISC86, NexBus, NxPCI, NxMC, and NxVL are trademarks of NexGen, Inc..

IBM, AT, and PS/2 are registered trademarks of International Business Machines, Inc. Intel is a registered trademark of Intel Corporation. i386, i387, i486 and Pentium are trademarks of Intel Corporation.  Tri-state is a registered trademark of National Semiconductor Corporation. VL-Bus is a trademark of Video Electronics Standards Association.

Restricted Rights and Limitations

Use, duplication, or disclosure by the Government is subject to restrictions set forth in subparagraph (c)(1)(ii) of the Rights in technical Data and Computer Software clause at 252.2777-7013

Contents

Introduction

Processor Recognition

Recognition Program

CPUID Instruction

List of Figures

Figure 1 Flow Chart Of Processor Recognition Routine

Figure 2 CPUID.ASM

Figure 3 CPUCLK.ASM

Figure 4 NXCPU.C

List Of Tables

Table 1 NexGen Online Resources

Table 2 NexGen Processor Signatures

# Introduction

The NexGen Nx586Ò processor offers a powerful and affordable alternative to Intel’s PentiumÔ processor.  The Nx586 processor is a 5th generation processor with full x86 binary compatibility.  In order to properly identify NexGen processors and their features, NexGen is providing software that performs these functions.  As the number of alternatives in the x86 market increases it is important for software to be able to identify the features and performance level associated with a given processor.

This application note explains the method for identifying a NexGen processor and its features.  It provides a software routine necessary to perform this function.  It also explains how this routine and the method it describes can be used by software developers in BIOS code, software applications, and utilities to properly identify current and future NexGen processors.

# Processor Recognition

To best leverage existing processor recognition routines and minimize the effort for software developers, the NexGen processor recognition code is designed as an extension of the processor recognition code published by Intel in AP-485, Intel Processor Identification With the CPUID Instruction Application Note.

Figure 1 provides a flow chart of the process used to identify different processors.  The areas in gray are those added to recognize NexGen processors.  Note that there are two code additions to recognize NexGen processors.

The first is for older Nx586 processors. Since all older Nx586s use the same register and flag implementation as the 80386, Intel’s recognition code will identify the Nx586 as an 80386.  To identify the additional performance and features available, NexGen has developed code to distinguish an Nx586 processor from an 80386.  This is done by using the fact that the 80386 and Nx586

Figure 1 Flow Chart Of Processor Recognition Routine

processor affect the ZF flag (bit 6 of EFLAGs) differently as a result of a DIV instruction and specific operands.  The Nx586 does not change the value of the ZF flag during the DIV while the 80386 changes the ZF flag according to the result of the execution.

The second piece of code uses the CPUID instruction to determine the type of processor and its features.  NexGen supports the CPUID in newer versions of the Nx586 processor.  To determine if the CPUID instruction is supported, software must test the ID bit (bit 21) in EFLAGS to determine if its value can be changed.  The code example in this application note includes this test.  Once the software determines that the CPUID instruction is available, it can execute this instruction to determine the processor’s vendor, family, type, features, and other useful information.  The section entitled “CPUID Instruction” describes the functionality of the CPUID instruction.

# Recognition Program

The following code examples (Figure 2, Figure 3, and Figure 4) enable software to identify NexGen processors and the features that they support.  These routines can be integrated with the recognition routines for other x86 processors to provide a complete solution for processor recognition.

An electronic copy of the code can be obtained from NexGen.  The code is available as a self-extracting zip file, CPUID52.EXE.  Table 1 provides the paths available for obtaining the code from NexGen.

|  |  | file area | filename |
| --- | --- | --- | --- |
| BBS | (408)955-1839 | Techdesk | CPUID52.EXE |
| World Wide Web | http://www.nexgen.com | Support Desk | CPUID52.EXE |
| FTP | ftp.nexgen.com | Techdesk | CPUID52.EXE |

Table 1 NexGen Online Resources

Figure 2 contains the file “cpuid.asm”.  This file contains two routines, “_get_nxcpu_type” and “_get_nxfpu”, that have been written in assembly language.  The “_get_nxcpu_type” routine implements the code necessary to identify NexGen processors.  It first checks for the AC bit (bit 18) in EFLAGS.  If the AC bit is not writeable, it then tests the ZF FLAG result from the DIV instruction.  If the ZF FLAG is unchanged, it is an Nx586 processor.

If during the initial test, the AC bit is found to be writeable, the code immediately tests for the ID bit.  If it is writeable, the code executes the CPUID instruction to identify the vendor, family, model, and features associated with the processor.

The “_get_nxfpu” routine implements the code necessary to determine if a floating point processor is present when the CPUID instruction is not supported.  This routine tests for the presence of the floating point processor by testing the floating point status word. contains the file “cpuclk.asm”.  This file contains a single assembly language routine, “_Nx586_clock_rate” that determines the operating frequency for the Nx586 processor.  This routine calculates the CPU clock rate by determining the time elapsed to execute a known number of CPU clock cycles.  The frequency calculated to the nearest 1/10th MHz and is returned in the AX register as 10 times the number of MHz.

Figure 4 contains the file “nxcpu.c”.  This file contains the C language program that calls the “_get_nxcpu_type” and “_get_nxfpu” routines to identify the type of processor and  determine the presence of the numeric processor.  It then prints the results to the screen.  In addition, the program calls “_Nx586_clock_rate” to determine the processor’s operating frequency and display this information on the screen.  Finally, if the CPUID instruction is available, this routine displays the vendor identification string, the processor signature (family, model, and stepping), and the feature flags.

Figure 2 CPUID.ASM

page ,132

;****************************************************************************

;

;   NexGen, Inc.

;   1623 Buckeye Drive

;   Milpitas, CA 95035

;   Phone: (408)435-0202

;

;****************************************************************************

;****************************************************************************

; File: cpuid.asm

;	Revision: 1.0

;

; This sample file contains two routines: "_get_nxcpu_type" and "_get_nxfpu".

; "_get_nxcpu_type" identifies NexGen's processor and saves CPU information

; in the data segment, and "_get_nxfpu" tests if FPU is present.

;

;****************************************************************************

DOSSEG

.model 	small

.386

CPU_ID	macro

db	0fh, 0a2h

endm

NONE		equ	0

PRESENT		equ	1

Nx586		equ	5

UNKNOWN		equ	0

.data

public	_nxcpu

public	_cputype

public	_cpuid_flag

public	_vendor_id

public	_cpu_signature

public	_features_ecx

public	_features_edx

public	_features_ebx

public	_nxfpu

_nxcpu		db	NONE			;default to none

_cputype	db	UNKNOWN			;default to unknown

_cpuid_flag	db	NONE			;default to no CPUID

_vendor_id	db	"************"

_cpu_signature	dd	0

_features_ecx	dd	0

_features_edx	dd	0

_features_ebx	dd	0

_nxfpu		db	NONE			;default to none

fp_status	dw	0

NexGen_id	db	"NexGenDriven"

.code

;==========================================================================

; _get_nxcpu_type

;	This routine identifies NexGen's processor type in following steps:

;

;	if (no AC flag) {	//current Nx586 does not support AC flag

;		set ZF=1;

;		execute DIV to result a none zero value;

;		if (ZF=0) {	//ZF is changed

;			not a NexGen processor;

;			exit;

;		} else {	//Nx586 does not change ZF on DIV instruction

;			if (ID bit not writeable) {

;				CPU is Nx586 with no CPUID support

;			} else { 		//Nx586 with CPUID support

;				execute CPUID instruction;

;				save CPU information;

;			}

;		}

;	} else {

;		if (ID bit not writeable) {

;			not a NexGen processor;

;		} else {	//NexGen future processors support CPUID

;			execute CPUID instruction;

;			save CPU information;

;		}

;	}

;

;==========================================================================

public	_get_nxcpu_type

_get_nxcpu_type	proc	near

mov	byte ptr _nxcpu,PRESENT	; default to present

; test AC bit on EFLAGS register

mov	bx,sp		; save the current stack pointer

and	sp,not 3	; align the stack to avoid AC fault

pushfd			;

pop	eax		; get the original EFLAGS

mov	ecx,eax		; save the original EFLAGS

xor	eax,40000h	; flip AC bit in EFLAGS

push	eax		; save for EFLAGS

popfd			; copy it to EFLAGS

pushfd			;

pop	eax		; get the new EFLAGS value

mov	sp,bx		; restore stack pointer

xor	eax,ecx		; if the AC bit is unchanged

je	test_zf		;	goto second step

jmp	nx_future_cpu

test_zf:

; test ZF on DIV instruction

mov	ax,5555h	; init AX with a non-zero value

xor	dx,dx		; set ZF=1

mov	cx,2

div	cx		; Nx586 processor does not modify ZF on DIV

jnz	not_nx_cpu	; not a NexGen processor if ZF=0 (modified)

test_cpuid:

; test if CPUID instruction is available

; new Nx586 or future CPU supports CPUID instruction

pushfd			; get EFLAGs

pop	eax

mov	ecx,eax		; save it

xor	eax,200000h	; modify ID bit

push	eax

popfd			; save it in new EFLAGS

pushfd			; get new EFLAGS

pop	eax		;

xor	eax,ecx		; is ID bit changed?

jnz	cpuid_present	; yes

mov	byte ptr _cputype,Nx586	; no, current Nx586

jz	cpuid_exit	; stop testing

nx_future_cpu:

; all NexGen's future processors feature a CPUID instruction

mov	eax,ecx		; get original EFLAGS

xor	eax,200000h	; modify ID bit

push	eax

popfd			; save it in new EFLAGS

pushfd			; get new EFLAGS

pop	eax		;

xor	eax,ecx		; is ID bit changed?

jz	not_nx_cpu	; no, not a NexGen processor

cpuid_present:

; execute CPUID instruction to get vendor name, stepping and feature info

xor	eax,eax

CPU_ID

mov	dword ptr _vendor_id,ebx

mov	dword ptr _vendor_id[+4],edx

mov	dword ptr _vendor_id[+8],ecx

mov	bx,ds

mov	es,bx

mov	si,offset _vendor_id

mov	di,offset NexGen_id

mov	cx,12

cld

repe	cmpsb		; compare vendor ID string

jne	not_nx_cpu

mov	byte ptr _cpuid_flag,PRESENT

cmp	eax,1		; check highest level

jl	cpuid_exit

mov	eax,1

CPU_ID

mov 	_cpu_signature,eax

mov	_features_ecx,ecx

mov	_features_edx,edx

mov	_features_ebx,ebx

shr	eax,8

and	al,0fh

mov	_cputype,al

jmp	cpuid_exit

not_nx_cpu:

mov	byte ptr _nxcpu,NONE

cpuid_exit:

ret

_get_nxcpu_type	endp

;==========================================================================

; _get_nxfpu

;	This procedure identifies NexGen's floating point processor by

; testing the floating point status word.

;

;==========================================================================

public	_get_nxfpu

_get_nxfpu	proc	near

mov	_nxfpu, PRESENT		; default to present

fninit				; reset fpu status word

mov	fp_status,0aa55h

fnstsw	fp_status

mov	ax,fp_status

cmp	al,0

je	nxfpu_end

mov	_nxfpu, NONE

nxfpu_end:

ret

_get_nxfpu	endp

end

Figure 3 CPUCLK.ASM

page ,132

;*************************************************************************

;

;   NexGen, Inc.

;   1623 Buckeye Drive

;   Milpitas, CA 95035

;   Phone: (408)435-0202

;

;*************************************************************************

;*************************************************************************

; File: cpuclk.asm

;	Revision: 1.0

;

; This file contains a "C" callable routine:

;

;	1) _Nx586_clock_rate returns CPU clock rate in MHz*10 unit.

;	   (i.e. A value of 600 means 60.0 MHz)

;

; The routine returns the result in AX register. You need to declare

; the function prototypes in the C program as:

;

;	extern unsigned _Nx586_clock_rate (void);

;

; The _Nx586_clock_rate returns the clock rate in MHz*10 unit.;

;

; Notice: these routines are coded for SMALL memory model.

; You have to change the .MODEL directive if your C program is using

; a different memory model. For example, the following directive will

; make routines callable from a C program in the LARGE memory model.

;

;	.MODEL LARGE, C

;

; To assemble this file into an object file if you are using Microsoft

; Assembler (5.10 or later), type this command:

;

; 	masm cpuclk;

;

; If you are using Borland Turbo Assembler, type this command:

;

;	tasm cpuclk;

;

;

; Revision History:

;	1.0 - initial release

;

;*************************************************************************

.MODEL	small

.386p

.DATA

clkcnt	dw	16 dup (0)

.CODE

;==========================================================================

; _Nx586_clock_rate

; 	This routine calculates the CPU clock rate by reading the time

; elapsed on a known number of CPU clock cycles. The total number of

; clock cycles is obtained from the cycle number defference of two

; instruction loops; a long cycle loop (DIV EBX) and a short cycle loop

; (DIV BX). The time elapsed on executing these number of cycles is

; the time difference of the long and short loop.

;

;	This routine gets the time difference of the two cycle loops for

; five times, and calculates their average. Then, the routine computes

; the CPU clock rate in MHz*100 unit, rounds off the last digit, and

; return the clock rate in MHz*10 unit. The calling program (C program)

; has to convert it to MHz unit.

;

; Input: none

; Output: AX = clock rate in MHz*10 unit

;==========================================================================

PUBLIC	_Nx586_clock_rate

_Nx586_clock_rate	PROC

push	ds

push	es

mov	ax,@data

mov	ds,ax

mov	es,ax

ASSUME	DS:@data

mov	cx,1		; execute sub-routine once to make sure

;   cache hit

xor	al,al		; a dummy call to clock routine

call	getclk

mov	di,offset clkcnt

cld

mov	cx,5		; run clock detection for 5 times

nextcount:

push	cx

; for each time:

mov	cx,400		;  perform 400 short delay loops (DIV BX)

mov	al,1		;  select short cycle loop

call	getclk		;  get timer tic

mov	bx,ax		;  save it

mov	cx,400		;  perform 400 long delay loops (DIV EBX)

xor	al,al		;  select long cycle loop

call	getclk		;  get timer tic

sub	ax,bx		;  calculate time difference of the two loops

stosw			;  save it

pop	cx

loop	nextcount

xor	dx,dx		; init DX:AX

xor	ax,ax

mov	cx,5

mov	si,offset clkcnt

totalcount:			; sum the total time difference in DX:AX

add	ax,[si]

adc	dx,+0

inc	si

inc	si

loop	totalcount

mov	bx,5		; calculate average time difference of

div	bx		;  each loop

mov	bx,ax		; calculate clock rate in MHz*100 unit

mov	ax,64000	; freq=(total cycle/time elapsed)*100

mov	cx,1193		;     =(("DIV EBX" clock - "DIV BX" clock)*100

xor	dx,dx		;      *400 loops)/(time difference/1.193 MHz)

mul	cx		;      *100

div	bx		;     =((34-18)*100*400/(timer difference/

;      1193000))*100

;     =64000*1193/time difference

xor	dx,dx

add	ax,5		; round off the last digit

adc	dx,+0

mov	bx,10		; disgard the last digit

div	bx		; return clock rate in MHz*10 unit

pop	es

pop	ds

ret

_Nx586_clock_rate	ENDP

;=======================================================================

; Getclk

;

;	Get timer tics after executing one of two clock loops. The long

; clock loop performs 100 "DIV EBX" instructions, and the short clock

; loop executes 100 "DIV BX" instructions. A loop count is passed through

; CX register to extend the total delay time.

;

;	The time tics returned from long and short clock loops can be used

; for clock rate calculation. The time difference of two clock loops is

; exactly same as the total cycle difference of the two loops. The total

; cycle difference is:

;

;		(34-18)*100*loopcount

;

; The Nx586 processor uses 34 cycles to execute "DIV EBX" instruction, and

; uses 18 cycles to execute "DIV BX" instruction.

;

; Input: CX = loop count

;	 AL = 0 to select long clock loop

;	    = 1 to select short clock loop

;

; Output: AX = timer tic from timer 2

;=======================================================================

getclk	PROC	NEAR

push	si

push	di

push	bx

push	dx

mov	bl,al		; save cycle loop selection

xor	dx,dx

in	al,61h		; get current value from port 61h

and	al,0FCh		; disable Gate2 and Speaker Data

out	61h,al

mov	al,0B4h		; program timer 2 with mode 2 (rate generator)

out	43h,al

mov	al,0ffh		; init timer 2 starting count

out	42h,al		; write LSB

out	42h,al		; then MSB

in	al,61h		; read port 61h again

mov	di,ax		; save this value for later use (to disarm

;   timer 2

or	al,1		; enable timer 2 by enabling Gate2

cmp	bl,0		; check which loop to perform

je	long_loop

jmp	short_loop

long_loop:

cli			; disable interrupt

out	61h,al		; arm timer 2

mov	ebx,2		; set divisor to a simple value

xor	edx,edx		; init EDX:EAX

xor	eax,eax		; NOTE: the time spent on instructions after

;   arming the timer 2 and before the DIV loop

;   will be eliminated by calculating the

;   time difference of two cycle loops. (see

;   short_loop below)

next_long:

div	ebx		; perform "DIV EBX" for 100 times

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

div	ebx

dec	cx

jnz	next_long

jmp	end_loop

short_loop:			; select short cycle loop

cli			; disable interrupts

out	61h,al		; arm timer 2

mov	ebx,2		; init divisor to a simple value

xor	edx,edx		; init EDX:EAX

xor	eax,eax		; these three instructions are exactly same

;   the ones used in the long cycle loop

next_short:

div	bx   		; perform "DIV BX" for 100 times

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

div	bx

dec	cx

jnz	next_short

jmp	end_loop	; same JMP instruction as the one used in the

nop			;   long loop

nop

end_loop:

mov	ax,di		; retrieve saved value for port 61h

out	61h,al		; disarm timer 2

sti			; enable interrupt

in	al,42h		; read LSB from timer 2

xchg	ah,al

in	al,42h		; read MSB

xchg	ah,al

neg	ax		; total count elasped

pop	dx

pop	bx

pop	di

pop	si

ret

getclk	ENDP

END

Figure 4 NXCPU.C

//*************************************************************************

//

//   NexGen, Inc.

//   1623 Buckeye Drive

//   Milpitas, CA 95035

//   Phone: (408)435-0202

//

//*************************************************************************

//*************************************************************************

// File: nxcpu.c

//   Revision: 1.0

//

// This sample C program identifies Nx586 processor and prints its information

// according to the data saved by the external procedures "get_nxcpu_type",

// "get_nxfpu", and "cpuclk". If the CPUID instruction is available, the

// vendor ID, family ID, stepping number and features supported will be

// displayed.

//

// The first two external functions can be found in CPUID.ASM. The

// CPUID.ASM is assembled in SMALL model, and should be linked with this

// program.

//

// The third routine, "Nx586_clock_rate", is found in CPUCLK.ASM. It should

// also be assembled in SMALL model, and linked with this program.

//

// Revision History:

//	1.0 - initial release

//

//*************************************************************************

#include <stdio.h>

extern	char nxcpu;

extern	char cputype;

extern	char cpuid_flag;

extern	unsigned long cpu_signature;

extern	unsigned long features_ecx;

extern	unsigned long features_edx;

extern	unsigned long features_ebx;

extern	char nxfpu;

extern	void get_nxcpu_type(void);

extern	void get_nxfpu(void);

extern	int Nx586_clock_rate(void);

void main (void)

{

get_nxcpu_type();

get_nxfpu();

print_cpu_info();

}

print_cpu_info()

{

if (!nxcpu) {

printf ("This system does not have a NexGen processor.\n");

exit(-1);

}

printf ("This system has an ");

switch (cputype) {

case 5:

printf ("Nx586[R] processor ");

if (nxfpu)

printf ("and a floating point processor");

printf ("\n");

printf ("\nProcessor running at %d MHz\n", (int)Nx586_clock_rate()/10);

printf ("\n");

if (cpuid_flag)

print_id_info();		//print more CPU information

break;

default:

//reserved for future expansion

break;

}

}

print_id_info()

{

printf ("Vendor ID: NexGenDriven\n");

printf ("Processor Family: %x\n",(char)((cpu_signature>>8) & 0xff));

printf ("Stepping: %x\n",(char)(cpu_signature & 0xf));

printf ("Feature Flags: %x\n",(char)(features_edx & 1));

}

# CPUID Instruction

The CPUID instruction is an application level instruction that software can execute to identify the processor and its feature set.  It can be executed from any privilege level.  Software can use this information to tune functionality for the specific processor and its features.

Not all processors implement the CPUID instruction.  Before executing the instruction, software should first test to see if the instruction exists.  Existence of the CPUID instruction is indicated by the ID bit  (21) in the EFLAGS register.  If this bit is writeable, the CPUID instruction exists.

Opcode:   0F A2

Input:  EAX

Output:  EAX, EBX, ECX, EDX

Function:

EAX = 0:

EAX = Highest input value recognized by CPUID instruction

EBX, EDX, ECX = Vendor identification string

EAX = 1:

EAX = Processor signature

EBX = Reserved

ECX = Reserved

EDX = Feature flags

EAX > 1:

EAX = Undefined

EBX = Undefined

ECX = Undefined

EDX = Undefined

Highest Input Value:

The highest input value recognized by the CPUID instruction in the Nx586 or Nx686 is 1.  If an input value greater than 1 is used the values returned in EAX, EBX, ECX, and EDX are undefined.  Future processors may implement higher values and the results returned by these values will be defined at that time.

Vendor Identification String:

The vendor identification string identifies NexGen as the vendor for the CPU.  It does so by returning “NexGenDriven” in the EBX, EDX, and ECX registers.

EBX = 4778654Eh (GxeN)

EDX = 72446E65h (rDne)

ECX = 6E657669h (nevi)

Processor Signature:

The processor signature identifies the specific CPU by providing information regarding its type, family, model, and stepping revision.  The information is formatted as follows:

EAX[0:3] = Stepping Revision

EAX[4:7] =  CPU Model

EAX[8:11] =  CPU Family

EAX[12:31] =  Reserved

Feature Flags:

The feature flags indicate the existence or presence of specific features.  In most cases a “1” indicates the feature is present.  The following is an explanation of the feature flags currently defined.  Reserved bits will be used in the future for new features as they are added.

EDX[0] = Floating Point Unit (1 indicates floating point unit is present, 0 indicates no floating point unit)

EDX[1:31] =  Reserved

Note:  All registers and bits marked “Reserved” should return 0.

Table 2 NexGen Processor Signatures

| CPU Family | CPU Model | Stepping Revision1 | Description |
| --- | --- | --- | --- |
| 0101 | 0000 | xxxx | Nx586 Processor |
| 0110 | 0000 | xxxx | Nx686 Processor |

Notes:

1. Contact NexGen for specific stepping revision information.