LCTERM

This utility is a terminal emulation program used for communicating with LC2. Files can be down-loaded and up-loaded between LC2 and the IBM PC. A Status Line appears at the top of the screen to indicate the current operating parameters. The bottom line shows a menu of operations and each associated function key. The commands are as follows:

F1 : HELP - Brings up this help screen.

F2 : UPLD - Uploads a file. Pressing any key will close the file.

F3 : DWNLD- Downloads a file.

F4 : PORT - Allows changing the input serial port number (1,2,3 or 4).

F5 : BAUD - Allows changing the baudrate (300, 1200, 9600, 19200).

F6 : HIBIT- When on, will strip the high bit off characters displayed.

F7 : DIR  - Displays the directory of the current default disk drive.

F8 : STOP - Sends a CTRL-C to LC2 and resets the screen buffer pointers.

F9 : CLEAR- Clears the screen and status line messages.

F10: EXIT - Exit the LCTERM program.

The status line is also used to prompt for the parameters related to the above commands. For a more detailed instruction summary see the file LCTERM.TXT

LCTERM

Version 1.35        8/20/86

This utility is a terminal emulation program used for communicating with LC2. Files can be down-loaded and up-loaded between LC2 and the IBM PC. A Status Line appears at the top of the screen to indicate the current operating parameters. The bottom line shows a menu of commands and each associated function key. The commands are as follows:

F1 : HELP - Brings up the help screen.

The Help screen is a condensed version of these instructions.

Pressing the F1 key while in the terminal mode will clear the

screen and display the help screen. Pressing any key will clear

the screen and return you to the terminal mode.

F2 : UPLD - Uploads a file.

When the F2 key is pressed, the cursor appears at the "FILENAME"

field on the status line. Enter the filename you wish to use for

storing the information being received at the serial port. After

the filename is entered and the return key is pressed, you will

be prompted with "COMMAND:" on the main screen. The response to

this prompt is passed to LC2 to initiate a transfer of characters.

An example when in BASIC, is to enter "LIST" in response to the

"COMMAND:" prompt. This causes LC2 to list the program currently in

its memory which is then received and stored on disk by LCTERM. When

uploading from LC2, LCTERM opens a disk file with the name used

in the FILENAME field and will begin storing anything received

after the command is issued. The file is closed when any key is

pressed or when LC2 sends an "Ok" response followed by a carriage

return and a line feed character. Neither the "Ok" response or

the initial command sent to LC2 are stored in the file on disk.

F3 : DWNLD- Downloads a file.

Pressing the F3 key, causes the cursor to appear at the "FILENAME"

field on the status line. Enter the filename of the file which

contains the information you which to send to LC2. LC2 must first

be set up to receive a file by making sure a program is not presently

running and the NEW command has been issued. Press the F3 Key only

after the NEW command has been sent to insure LC2's memory is clear.

If a filename is entered for a file which does not exist, a FILE ERROR

message will appear in the status field and you will be returned to

the terminal mode (See F7 - DIR Below). The file must be an ASCII only

source file written in LC2 BASIC, otherwise "syntax error" messages

will appear on the screen. If the file is a FORTH source file it must

be an ASCII file and not a screen file. The NEW command is not used in

FORTH.

F4 : PORT - Allows changing the input serial port number ( 1, 2, 3, or 4 ).

Pressing the F4 Key will clear the present COM port number in the

PORT field of the status line and place the cursor at the beginning

of the field. Only a 1, 2, 3, or a 4 will be accepted as a response to

this field. The serial port corresponding to this value will then

be initialized with the baudrate specified in the BAUD field. When using

COM ports 3 and 4, the communications card must be set to the following

addresses and interrupts:

COM3   at  Hex Address 348  and IRQ 2

COM4   at  Hex Address 340  and IRQ 5

F5 : BAUD - Allows changing the baudrate (300, 1200, 9600, 19200).

Pressing the F5 key will clear the value in the BAUD field and

place the cursor at the beginning of the field. Only the valid

baudrates of 300, 1200, 9600, or 19200 will be accepted. The

serial port specified in the PORT field will then be initialized

to the selected baudrate.

F6 : HIBIT- When on, will strip the high bit off characters displayed.

This command toggles the value in the status field  HB  between

"OFF" and "ON". The default setting is "ON". Any characters being

received at the serial port will be displayed as a 7-bit ASCII

value when HB is "ON". Any characters transferred via the UPLD or

DWNLD commands will always have all 8-bits intact. Setting HB to

"OFF" allows the special graphics characters to be passed and

displayed on the terminal.

F7 : DIR  - Displays the directory of the current default disk drive.

Pressing the F7 key will clear the main screen and display a

directory listing of all the files on the default disk drive and

default directory. Hidden files are also displayed.

F8 : STOP - The STOP key sends a CTRL-C to LC2 to interrupt him when running

or listing a program and resets the communications buffer of LCTERM.

F9 : CLEAR- Clear resets the communication buffer of LCTERM and clears the

main screen and the status fields.

F10: EXIT - Exit the LCTERM program.

LCTERM DEFAULT PARAMETERS FILE

If you would like LCTERM to automatically set a specific port number,

baudrate and disable the introductory screen, create an ASCII file called

LCTERM.CFG which contains the following parameters. There are three

parameters; p for port, b for baud, h for help. Each parameter is followed

by an appropriate number and each parameter must be in the first space of

each line in the file. A typical example for setting the default port to

2, the baudrate to 19200 baud and no introductory message is:

p 2

b 19200

h 0

Setting h to anything but 0 will cause the signon message to be displayed.

Setting the wrong value for port or baudrate or leaving out one of these

parameters will cause LCTERM to prompt you for the value. You must specify

a parameter letter followed by a space and a number on each line or

a RUN TIME ERROR will occur and you will return to DOS. If this happens

check the LCTERM.CFG file and make the proper corrections. Then rerun LCTERM.

The file is not neccessary but is provided as a convenience to the user.

If LCTERM cannot locate the file LCTERM.CFG, an introductory message will

appear and you will be prompted for the port number and baud rate when

LCTERM starts up.