# 1.  Product Description

The EXM-SSD is an EXM expansion module containing a solid state Flashdrive, an IDE floppy disk controller and a PC/AT controller for external floppy disk drives.  It provides a fully functional solid state hard disk in a single EXM slot form factor.

A front-panel connector is provided for addition of one or two 3.5" or a 5.25" external floppy disk drive(s).  There are also two indicator LEDs; one indicates the presence of +5V power and the other indicates that an access to the hard disk is in process.

The EXM-SSD expansion module uses the Seagate SunDisk Flashdrive and is fully  PC-compatible at the hardware and software level. The SunDisk Flashdrive is logically equivalent to an IDE disk drive and is controlled by a standard IDE controller. No special Flash file system software is required.

The construction and definition of the EXM-SSD prohibits its use in several logical and mechanical configurations.  The contexts in which an EXM-SSD may not be used are the following:

·	In the same system with an EXM-9 disk controller unless the EXM-SSD and EXM-9 are not enabled at the same time

·	In the same system with an EXM-3 disk controller or EXP-MS40 unless at most one floppy controller is enabled at the same time

·	In the same system with another EXM-SSD, unless at most one hard disk and one floppy controller are enabled at the same time

·	In the same system with an EXM-MX, unless at most one hard disk and one floppy controller are enabled at the same time

·	In the same system with an EXP-MX mass storage module

·	In an EPC-2, EPC-6, or EPC-7

·	In the upper EXM slot of an EPC-4 or EPC-5

## Specifications

Table 1 defines the environmental and electrical specifications and performance benchmarks of the EXM-SSD. Note that the Flash memory technology is considerably more rugged than traditional rotating magnetic storage disks.

| Characteristic |  | Value |
| --- | --- | --- |
| Performance |  |  |
| Throughput | sustained reading | up to 650 KBytes per second |
|  | sustained writing | up to 60 KBytes per second |
| Endurance |  | greater than 55,000 cycles |
| Electrical |  |  |
| Current | writing | 350 mA |
|  | erasing | 300 mA |
|  | reading | 150 mA |
| Environmental |  |  |
| Temperature | operating | 0° to 60° C (* see below) |
|  | storage | -20° to 85° C |
| Humidity | operating | 8% - 95% (non-condensing) |
|  | storage | 8% - 95% (non-condensing) |
| Vibration | operating | 15 Gs peak-to-peak max |
|  | storage | 15 Gs peak-to-peak max |
| Shock | operating | 500 Gs max |
|  | storage | 500 Gs max |

Table 1. EXM-SSD Specifications.

* Maximum elevation 40,000 ft.