# 2.  Installation

Before installing the EXM-SSD, unpack and inspect it for shipping damage.

*	do not remove the module from its anti-static bag unless you are in a static-free environment.  the exm-ssd, like most other electronic devices, is susceptible to esd damage.  esd damage is not always immediately obvious, in that it can cause a partial breakdown in semiconductor devices that might not immediately result in a failure.

*	ensure that the installation process as described herein is also performed in a static-free environment.

## Before Installation

Before inserting the EXM-SSD into the system, check the drive type label on the side of the unit.  Write down the appropriate drive type to use for this module.  You will need this information later.

If you are not going to install an external floppy drive, you can skip the remainder of this section and proceed to Installing the EXM-SSD on the next page.

The EXM-SSD provides the option of supplying +5VDC to the floppy drive through the ribbon cable.  Before connecting an external floppy drive, check the drive specifications to determine if your particular drive can take advantage of this feature.  There is a three-pin header near the rear of the EXM-SSD circuit board labeled "JP1 enable +5 Ý FDD" (see diagram, next page). There is a middle pin, a pin labeled OFF, and a pin labeled ON.  If you plan to provide external power to the floppy drive, connect the middle pin to the OFF pin.  If you plan to provide power through the ribbon cable, connect the middle pin to the ON pin.  For more detailed information about this feature, see the section Connectors on page 13.

*	providing power through the ribbon cable to a drive that cannot correctly use that power may damage the drive and will probably damage the exm-ssd.

The only valid configuration when the jumper connects the ON pin is:

·	Single floppy being used as A:

·	Straight ribbon cable

·	Floppy is configured to expect +5VDC through the ribbon cable

µ §

Figure 1. Floppy Power Jumper Location.

## Installing the EXM-SSD

*	make sure that power to your system is off.  the exm-ssd is not designed to be inserted or removed from a live system.

*	handle the module with care, avoiding sudden drops and jolts.  insert it with adequate continuous force rather than tapping or hammering on it.

*	when inserting the exm, avoid touching the circuit board, and make sure the environment is static-free.

Installation of the EXM-SSD into an EXM carrier is straightforward:

o	If you intend to use the built-in EXM carrier on an EPC-4 or EPC-5, the 
EXM-SSD can only be inserted in the lower EXM slot.

o	If the EPC is not already installed, insert the EPC into the chassis and press firmly on the front panel until you feel the EPC seat into the rear connector(s).

o	Remove a blank EXM panel from the carrier (by unscrewing the thumbscrews) and insert the EXM-SSD into the EXM card guides. Firmly press the 
EXM-SSD front panel to ensure that the module is properly seated in the rear connector and secure it with the thumbscrews.

## Adding an External Floppy Drive

One or two external floppy drives can be used with the EXM-SSD by attaching a ribbon cable from the drive to the 34-pin front-panel connector.  Most standard 3.5" and 5.25" drives can be used.

Floppy disk drives normally come configured as drive select 2.  A single drive connected with a straight ribbon cable will be addressed as drive A:.  A second drive (B:), configured with the same drive select, can be connected using a standard AT-type daisy-chain twisted ribbon cable (pins 10-16 are "twisted") that swaps the drive address lines for drive B:.

Some newer floppy drives can be powered through the ribbon cable.  These drives connect +5VDC through pins 7, 9, and 11 of the ribbon cable.  Other drives expect these pins to be Ground.  The EXM-SSD supports both types of drives.  However, care must be taken such that +5VDC and Ground are not connected together.  In particular, be careful with floppy drives that contain jumpers to configure whether +5VDC or Ground is connected to these pins.  If you are unsure which type of drive you have, or if you don't know how the jumpers should be configured for a specific drive, consult the manual for your particular drive before connecting the drive to the EXM-SSD.

The EXM-SSD circuit board contains a field replaceable fuse protecting the power circuit to the floppy drive.  If the fuse is blown, the floppy disk controller and the hard disk will continue to operate, but the +5VDC will not be available on the connector and both of the front-panel LEDs will not illuminate.

*	if the jumper is set to the on position and the floppy requires ground on these pins, the fuse will blow.

*	if the jumper is set to the on position, using a twisted cable will blow the fuse.  do not use a twisted cable if you are trying to power the drive through the cable.

If your floppy drive has jumpers or switches, follow these guidelines:

·	If numbering on your drive starts at 1 and the drive has a DS (drive select) jumper or switch, set DS to 2.  If numbering on your drive starts at 0 and the drive has a DS jumper or switch, set DS to 1.

·	If the drive has an MM/MS (motor control) jumper or switch, set it to MS (motor control derived from drive-select signal).

·	If the drive has a SR/DC (disk change) jumper or switch, set it to DC.

·	If the drive is 1.2 Mbyte, set to dual speed.  Check your drive manual for details.

## Configuration

To use the EXM-SSD, you will need to perform several steps to configure it 
logically into your system and prepare the hard disk for use:

·	Enable it as an EXM module

·	Specify the hard disk type in the CMOS setup

·	If using an external floppy drive(s), specify its (their) type in the CMOS setup

·	Partition and format the hard disk for your operating system

### The EPC BIOS

The BIOS in the EPC to which the EXM-SSD is connected needs to be configured to enable the hard disk and specify the hard disk and floppy disk drive type(s).  For the hard disk, the type identifies the basic hard disk parameters: the number of cylinders, heads, and sectors in the hard disk assembly.  Follow the instructions in the EPC reference manual for your specific EPC model to invoke the BIOS setup function. This is typically done by pressing the CTRL-ALT-ESC keys simultaneously.  EPC system configuration procedures differ slightly depending on BIOS versions.

### Enabling the EXM module

Once in the setup program, a menu displays specifying which function keys are available for further configuration.  Press the F2 function key to invoke the EXM menu.  The screen display resembles the figure below.

ID OB1 OB2       
 Slot 0    FF  00  00
      1    DB  C1  00
      2    7D  05  00
      3    DE  01  00
      4    F5  05  00
      5    DC  F5  91

Figure 2. EXM Setup Screen.

The EXM Setup Screen displays the EXM configuration data (in hexadecimal) stored in nonvolatile memory which the EPC uses at power-up to recognize and configure each installed EXM.  An example of configuration data for the EXM-SSD is shown in bold in slot 2. The displayed data shows SLOT, ID, OB1 and OB2.  These are defined as follows:

SLOT	indicates the slot in which the EXM is installed.  See the diagram below to determine which EXM slot the EXM-SSD occupies.  Note that dashed lines indicate EXM slots that may not be available on all systems.

EXM Slots

µ §

Figure 3. EXM Slot Numbering.

ID 	is a hard-wired ID value.  Each EXM has a unique ID value.

OB1/OB2 	are two bytes of option information.

All slots not occupied by an EXM module show an ID of FF and OB1/OB2 of  00 00  indicating no EXM is present.

To add or change an EXM configuration, use the cursor keys (arrows) to move between the fields on the screen.  Move the cursor to the appropriate slot entry and type in the correct value.

The ID for the EXM-SSD should be set to 7Dh.

OB1 is a hexadecimal value derived by computing the following:

| Bits | Description |
| --- | --- |
| 00 | EXM-SSD disabled |
| 03 | Floppy controller enabled, IDE interface disabled |
| 05 | IDE interface enabled, floppy controller disabled |
| 07 | IDE interface and floppy controller enabled |

Table 2. OB1 Bit Definition.

For example, a typical OB1 value is 05h (00000101) indicating the IDE interface is enabled and the floppy controller is disabled.

OB2 is not used and should be set to 00.

Press F10 to save this data and return to the main setup screen.

### Specify the Hard Disk Type

A separate screen exists for specifying the fixed (hard) disk type.  Check the function key menu to determine the appropriate function key to bring up the Fixed disk menu.  The screen is similar to the one below.

Fixed Disk Menu

 Fixed Disk Drive C: None

 Fixed Disk Drive D: None

Figure 4. Fixed Disk Menu Display.

If you are using the hard disk in the EXM-SSD, you should first toggle to the AT option for drive C.  Selection of this option causes a type line to appear.  The type line allows you to select among a large variety of predefined drive types, or to specify the detailed parameters of a drive not defined by a predefined type.  Toggle to the type number that matches the type number on the sticker on the side of your drive.

Some drives may not appear in the BIOS list. In this case, most BIOS versions include user-editable drive types (type 48 and 49) that allow you to manually enter the drive's parameters. When using the user-editable drive types use the parameters for cyclinders, heads and sectors found on the sticker on the side of your drive.

Fixed disk Drive D: should be set to NONE (or SCSI if you are using the 
EXP-MS40 or the EXM-3).  If you are using both the EXM-SSD and a SCSI hard disk with the EXM-3 or EXP-MS40, the EXM-SSD hard disk must be drive C:.  After selecting the appropriate values, press F10 to save this data and return to the main Setup screen.

After the Fixed disk(s) have been configured, press F10 to save the data or ESC to ignore the changes.  In either case you will be returned to the main setup screen.

### Specify the Floppy Drive Type

Finally, the main setup menu provides selections for the diskette drives.  If you have no external diskette drives, set both selections to NONE.  If you have an A: drive or both floppy drives, toggle through the selections to specify the one matching your floppy drive(s).

When you are sure that the CMOS Setup data is correct, press F10 to save this data.  Then press F5 to confirm the data and reboot.

### Disk Partitioning and Formatting

The EXM-SSD is shipped from the factory with no partition or high-level formatting.  The hard disk must be partitioned and formatted for your operating system before you can load any software.

Partitioning is the process of building the primary data structures on the hard disk that define the physical characteristics of the drive and divide the disk into one or more sections.  Formatting (also called high-level formatting) is the process of actually building a file system on a disk drive partition - basically setting up each partition so that it "looks like" a file system.

Partitioning and Formatting for DOS

The following procedure requires a floppy drive be installed.

The process for MS-DOS is described here.  Other operating systems have similar procedures, with different details.

1)	Boot the system using the operating system SETUP diskette.

2)	You will be asked several questions.  Follow the instructions on the screen until you see the following prompt:

Allocate all free hard disk space for MS-DOS

Allocate some free hard disk space for MS-DOS

Do not allocate free hard disk space for MS-DOS

The first option will be highlighted.  You can allocate ALL the hard disk space by pressing ENTER.  The SETUP program will partition the disk and reboot the system.  The SETUP program will be re-invoked and automatically start formatting the disk.

Note that you may choose to "Allocate some free hard disk space for MS-DOS" to allow multiple partitions on the disk.   If you do, the process is slightly different.

3)	The remainder of the installation process is automatic.  Follow the instructions on the screen.

## Replacing the Fuse

*	make sure that power to your system is off.  the exm is not designed to be inserted or removed from a live system.

*	handle the module with care, avoiding sudden drops and jolts.  insert it with adequate continuous force rather than tapping or hammering on it.

*	when handling the exm, avoid touching the circuit board, and make sure the environment is static-free.

The EXM-SSD has a socketed fuse (field replaceable) located at the bottom of the circuit board immediately behind the hard disk (see Figure 1, page 4).  Should the fuse blow, follow these instructions to replace it.

The replacement fuse is a Littlefuse 272-002 (or equivalent 2A fuse).

*	to prevent esd damage to the module, ensure that the replacement process described is performed in a static-free environment and you are properly grounded.

To replace the fuse,

o 	Ensure that power to the system is OFF.

o 	Remove the EXM-SSD by unscrewing the thumbscrews and gently pulling forward.

o	Place the EXM-SSD on a static-free work surface, with the component side up and the front panel to your right.

o	Remove the old fuse by pulling straight up with continuous pressure.  Do not jerk on the fuse as this may damage the sockets or the circuit board.

o	Insert the new fuse in the sockets.  Visually inspect the fuse to verify that both pins are completely seated in the sockets.

o	Replace the EXM-SSD module in the original EXM slot and secure by tightening the thumbscrews.

## Connectors

The EXM-SSD front panel contains a latching 34-pin ribbon-cable connector for attachment of one or two external floppy drives.

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |

µ §

| 1 | Ground | 2 | -DENS |
| --- | --- | --- | --- |
| 3 | not used | 4 | not used |
| 5 | not used | 6 | not used |
| 7 | Ground or Vcc | 8 | -IDX |
| 9 | Ground or Vcc | 10 | -MO2 |
| 11 | Ground or Vcc | 12 | -DS1 |
| 13 | Ground | 14 | -DS2 |
| 15 | Ground | 16 | -MO1 |
| 17 | Ground | 18 | -DIRC |
| 19 | Ground | 20 | -STEP |
| 21 | Ground | 22 | -WD |
| 23 | Ground | 24 | -WE |
| 25 | Ground | 26 | -TRQ0 |
| 27 | Ground | 28 | -WP |
| 29 | Ground | 30 | -RDD |
| 31 | Ground | 32 | -HS |
| 33 | Ground | 34 | -DCH |

Table 3. 34-Pin Floppy Connector Pin-Out.

Note that pins 7, 9, and 11 have a dual definition.  If the jumper labeled "JP1 enable +5 Ý FDD" is installed connecting the middle pin to the OFF pin, signals 7, 9, and 11 on the front panel floppy connector are Ground.  If the jumper is installed connecting the middle pin to the ON pin, signals 7, 9, and 11 are +5VDC.

Mating cable connectors for the front-panel connector include the following (or equivalents):

3M 3414-x634 (x meaning don't care)	T&B 622-3441

T&B 636-3441				AMP 499485-8

## Trouble Shooting

| Symptom | Possible Cause | Solution |
| --- | --- | --- |
| Cannot access hard drive and both front-panel LEDs are off | Most likely cause is absence of power to the EXM-SSD. | Check seating of the module. |
| Cannot access hard drive and +5V front-panel LED is on | BIOS has not enabled the EXM-SSD The fixed-disk type assumed by the BIOS is incorrect. There is another disk controller in the system interfering with the EXM-SSD. | Check the entry in the BIOS setup screen EXM menu. Check the drive type in the BIOS setup Fixed disk menu. Reread the configuration rules in Chapter 1. |
| Cannot boot from EXM-SSD. | The hard disk may not have been formatted as a system disk. Wrong drive type in the BIOS Setup. The partition may not be set Active. | See your operating system manual for details on making a system disk. Compare the BIOS setup with the drive type sticker. Use the partitioning function to set Active. |

Table 4. Troubleshooting Guidelines.

| Symptom | Possible Cause | Solution |
| --- | --- | --- |
| Cannot access external floppy drive(s) | A: or B: drive has not been correctly configured in the BIOS. Improper or faulty cable. Incorrect jumper/switch settings on the drive. Improperly formatted or unformatted diskette. | Invoke your BIOS setup program and check the type defined for this drive. Verify cable is good or replace with known good cable. Consult your drive manual for correct settings. Re-format the floppy disk. |
| Cannot boot from floppy drive. | The floppy disk may not have been formatted as a system disk. The BIOS definition of your floppy drive may be incorrect. | See your operating system manual for details on making a system disk. Check the BIOS setup screen to verify the correct floppy drive entry. |
| No power to external floppy drive | If you are powering the drive from the EXM-SSD, the jumper in the EXM-SSD may not be correct. If the floppy drive is powered from an external source, the power source may not be turned on or plugged in. Improper or faulty cable. The +5VDC fuse has been blown (indicated by both LEDs being off). | Connect the middle pin to the ON pin (See diagram, page 4) Verify that the external power source is functioning correctly and the power connector to the floppy is fully seated. Verify cable is good or replace with good cable. Replace the fuse (see section Replacing the Fuse, page 12). |

Table 4. Troubleshooting Guidelines, continued.

NOTES