# 3. Programming Interface

This chapter contains information needed to write custom software drivers for the EXM-SSD.  Anyone using the EXM-SSD as a standard PC/AT compatible hard disk and floppy disk controller can ignore this chapter.

The EXM-SSD defines the following registers in the I/O space.

|  | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 | I/O port |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Device ID Register | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Config Option Byte 1 Reg |  | IDEN | FDEN | CDEN | 102 |
| --- | --- | --- | --- | --- | --- |

| Data Register |  | 1F0 |
| --- | --- | --- |

| Error / Write Precomp Reg |  | 1F1 |
| --- | --- | --- |

| Sector Count Register |  | 1F2 |
| --- | --- | --- |

| Sector Number Register |  | 1F3 |
| --- | --- | --- |

| Cylinder Low Register |  | 1F4 |
| --- | --- | --- |

| Cylinder High Register |  | 1F5 |
| --- | --- | --- |

| SDH Register |  | 1F6 |
| --- | --- | --- |

| Status / Command Register |  | 1F7 |
| --- | --- | --- |

| Floppy Operations Register |  | 3F2 |
| --- | --- | --- |

| Floppy Command Register |  | 3F4 |
| --- | --- | --- |
| Floppy Data Register |  | 3F5 |

| Alt Status / Digital Out Reg |  | 3F6 |
| --- | --- | --- |

| Drive adr / Floppy Ctl Reg |  | 3F7 |
| --- | --- | --- |

The first two are standard EXM registers for device identification and configuration.  The EXM-SSD responds to accesses to these ports only if EXM expansion interface line -EXMID is asserted.  An 8-bit read from I/O address 100h returns the value 7D, the device ID of the EXM-SSD.  A read/write configuration register appears at I/O address 102h.  The defined bits are the following:

IDEN specifies whether the IDE interface and hard drive is enabled (1) or disabled (0).  If IDEN and CDEN are set, the I/O addresses 1F0-1F7 and 3F6-3F7 are active.

FDEN specifies whether the floppy diskette controller is enabled (1) or disabled (0).  If FDEN and CDEN are set, the I/O addresses 3F2-3F7 and the floppy diskette controller are active.

CDEN specifies whether the EXM-SSD is enabled (1) or disabled (0).  If disabled, the EXM-SSD will not respond to the 1F0-1F7 and 3F2-3F7 I/O addresses; it will only respond to reads from I/O port 100h and reads and writes from I/O port 102h, and then only if EXMbus line -EXMID is asserted.

Because of the IBM PC/AT architecture, both the IDE interface and the floppy controller use address 3F7; the IDE interface defines bits 0-6 and the floppy controller defines bit 7.  These individual bits are activated by the IDEN and FLEN enable bits.  For instance, if FLEN=0, bit 7 of I/O address 3F7 is not driven by the EXM-SSD, meaning that another floppy controller at address 3F7 can be active in the system.

For an explanation of the IDE and floppy drive registers, refer to documentation on any standard PC/AT fixed-disk and floppy drive interface.