;start(Preventive Maintenance)

Preventive Maintenance

��`IMPORTANT`������������������������������������������������������Ŀ

�  Perform these preventive procedures once every twelve months.  �

�������������������������������������������������������������������

^Base Unit/System Unit^

^Cash Drawer^

^Display^

^Keyboard^

^Keylocks^

^Model 1 or 2 Printer^

^Model 3 or 4 Printer^

^Helpful Hints^

;end(Preventive Maintenance)

;start(Base Unit/System Unit)

;title() [  Preventive Maintenance for the Base Unit/System Unit  ]

Vacuum the vent grills.

Check the cables and the power cord for chafing, frayed insulation,

cracks or breaks.

Install the store loop cable retainer (P/N 96X4783) if needed consult

RETAIN tip 29).

;end(Base Unit/System Unit)

;start(Cash Drawer)

;title() [   Preventive Maintenance for the Cash Drawer   ]

Check the cash drawer rails.

Use the rail repair kit if the rails are damaged (P/N 25F6269).

;end(Cash Drawer)

;start(Display)

;title() [   Preventive Maintenance for the Display  ]

Clean the display covers.

Check for broken covers and missing mounting hardware.

Replace if necessary.

;end(Display)

;start(Keyboard)

;title() [   Preventive Maintenance for the Keyboard   ]

Vacuum the keyboard.

Pass the MSR Cleaning card P/N 6019483 through the reader

a few times.

Run the keyboard test.

Clean the keyboard cover and the keytops.

;end(Keyboard)

;start(Keylocks)

;title() [   Preventive Maintenance for the Keylocks   ]

Apply 3 drops of silicon lubricant P/N 96X4791 into the key

opening of the keylocks in the cash drawer, keyboard and printer.

;end(Keylocks)

;start(Model 1 or 2 Printer)

;title() [   Preventive Maintenance for the Model 1 or 2 Printer   ]

Run the Stand-Alone Printer Tests.

Remove the bottom cover and vacuum the printer from the top and bottom.

Check the tear bar.

Clean the home, document insert, journal and cover sensors using

canned air (P/N 95F2200 - refill and P/N 95F2201 - valve).

`(P/N 2648326 is for Canned Air and is half the price).

Clean the print head.  ^Cleaning the Print Head^

Note: `Do not ` use the silicon oil to lubricate anywhere else in

the printer

Lubricate the carriage shaft, wear shoe skid-plate and the print head

guide rod shaft bearings with IBM #6 oil.

Clean the teeth of the toggle gear.

Replace the carriage shaft wiper and saturate it with IBM #6 oil.

;end(Model 1 or 2 Printer)

;start(Model 3 or 4 Printer)

;title() [   Preventive Maintenance for the Model 3 or 4 Printer   ]

Clean dust fuzz balls, debris and chad from the printer.

This will significantly increase the life of the printer.

Clean dust and debris from the following sensors using canned air.

- Home sensor

- Cover sensor mod 3 only

- Document insert (DI) Upper & Lower sensors (journal side).

NOTE: After you clean the DI sensors the `sensor sensitivity` needs to be

reset. To `reset` press the Feed Up, Feed Down, and Green button

at the same time, open the cover, wait for the line feeding to

stop and close the cover.

Clean Lead Screw and black carrier shafts with a clean dry cloth.

`DO NOT` lubricate the Lead Screw or the black carrier shafts.

Check to see if the print wires are snagging the ribbon. This

situation can cause home errors and require the printhead be

replaced.

Check the ribbon drive clutch for proper (one way) operation.

Remove the ribbon and wipe off the ink from the tip of the print head.

Ink can collect on the print head and smudge the forms.

Check/Clean the DI rollers using alcohol. Press Up or Down feed to

move rollers.

Check Upper Forms Entry Shield for even clearance across platen

face and make sure the shield is not trapped between the Platen

and the Print Head Assembly.

Check that the forms compensation system moves freely by feeding

a thick form. The print head should move back as the feed rollers

open up.

`Note:` Replacing or adjusting these parts is a Factory Only Repair.

Check blue idler rollers (2) and paper bucket rollers (4) for free

movement/rotation. Lubricate with #6 oil.

Clean the debris around the platen.

Oil cutter blade pivots with #6 oil. Wipe off excess oil.

Check the metal bails, in both the receipt and journal stations

that hold the paper against the platen, set flush against the

platen, verify paper loads correctly.

;end(Model 3 or 4 Printer)

;start(Helpful Hints)

;title() [   Preventive Maintenance Helpful Hints   ]

The microcode in the printer will allow a change in horizontal and

vertical alignment. This will compensate for wear in the printer so

if the print line appears to be misaligned, use the appropriate

^Printer Adjustments^ procedure for your terminal.

If the printer is reporting false home errors, the problem may be

the transport motor, helix assembly and nut.

False journal out of paper conditions which require the cover to

be opened even though paper is present, can be caused by the journal

slip clutch or the O-ring.

;end(Helpful Hints)

;start(Cleaning the Print Head)

;file() repair.prt Print Head Cleaning Procedure

;end(Cleaning the Print Head)

;start(Printer Adjustments)

;file() repair.prt

;end(Printer Adjustments)